# RISC-V 5-Stage Pipelined Processor (Verilog)

A Verilog implementation of a simplified 32-bit RISC-V-style pipelined processor with a 5-stage pipeline and a testbench for functional simulation.

# Overview

This project implements a basic pipelined RISC processor that supports a custom instruction set. The design uses a two-phase clock (clk1 and clk2) to separate pipeline stages and avoid data hazards between adjacent stages.

# MIPS32

- 32 x 32 bit GPRs [R0 to R31]

- R0 hardwired to logic 0

- 32 bit Program Counter (PC)

- No flag registers (carry, zero, sign etc)

- Few Addressing Modes

- Only Load and Store instructions can access memory

- We assume memory word size is 32 bits (word addressable)

# Pipeline Stages

| Stage | Clock | Description |
| --- | --- | --- |
| IF – Instruction Fetch | clk1 | Fetches instruction from memory; handles branch redirection |
| ID – Instruction Decode | clk2 | Decodes instruction, reads registers, sign-extends immediate |
| EX – Execute | clk1 | ALU operations, address calculation, branch condition evaluation |
| MEM – Memory Access | clk2 | Loads/stores data from/to memory |
| WB – Write Back | clk1 | Writes results back to the register file |

# Supported Instructions

R-Type (Register-Register ALU)

| Opcode | Instruction | Operation |
| --- | --- | --- |
| 000000 | ADD Rd, Rs1, Rs2 | Rd = Rs1 + Rs2 |
| 000001 | SUB Rd, Rs1, Rs2 | Rd = Rs1 - Rs2 |
| 000010 | AND Rd, Rs1, Rs2 | Rd = Rs1 & Rs2 |
| 000011 | OR Rd, Rs1, Rs2 | Rd = Rs1 ¦ Rs2 |
| 000100 | SLT Rd, Rs1, Rs2 | Rd = (Rs1 < Rs2) ? 1 : 0 |
| 000101 | MUL Rd, Rs1, Rs2 | Rd = Rs1 * Rs2 |
| 000110 | HLT | Halt execution |

I-Type (Immediate / Load / Store / Branch)

| Opcode | Instruction | Operation |
| --- | --- | --- |
| 001000 | LW Rd, offset(Rs) | Rd = Mem[Rs + offset] |
| 001001 | SW Rs2, offset(Rs1) | Mem[Rs1 + offset] = Rs2 |
| 001010 | ADDI Rd, Rs, imm | Rd = Rs + imm |
| 001011 | SUBI Rd, Rs, imm | Rd = Rs - imm |
| 001100 | SLTI Rd, Rs, imm | Rd = (Rs < imm) ? 1 : 0 |
| 001101 | BNEQZ Rs, offset | if (Rs != 0) PC = PC + offset |
| 001110 | BEQZ Rs, offset | if (Rs == 0) PC = PC + offset |

# Instruction Encoding

[31:26] – Opcode (6 bits)

[25:21] – Source Register 1 / Rs (5 bits)

[20:16] – Source Register 2 / Rd (5 bits)

[15:11] – Destination Register (RR-type, 5 bits)

[15:0]  – Immediate / Offset (16 bits, sign-extended)

# Branch Handling

Branch resolution occurs at the EX stage. When a branch is taken, the pipeline flushes the incorrectly fetched instructions using the TAKEN_BRANCH flag, and the PC is redirected to the computed branch target address.

# Simulation

Prerequisites

- Icarus Verilog or any compatible Verilog simulator (ModelSim, Vivado, etc.)

- GTKWave (optional, for waveform viewing)

Running with Icarus Verilog

# Compile

iverilog -o riscv_sim riscv.v

# Run simulation

vvp riscv_sim

# View waveforms

gtkwave mips.vcd

# Test Program

The testbench loads the following program into memory to verify basic arithmetic:

Mem[0] = ADDI R1, R0, 10    # R1 = 10

Mem[1] = ADDI R2, R0, 20    # R2 = 20

Mem[2] = ADDI R3, R0, 25    # R3 = 25

Mem[3] = OR   R7, R7, R7    # NOP (dummy)

Mem[4] = OR   R7, R7, R7    # NOP (dummy)

Mem[5] = ADD  R4, R1, R2    # R4 = R1 + R2 = 30

Mem[6] = OR   R7, R7, R7    # NOP (dummy)

Mem[7] = ADD  R5, R4, R3    # R5 = R4 + R3 = 55

Mem[8] = HLT

Dummy NOP instructions (OR R7, R7, R7) are inserted to handle pipeline data hazards manually, since this implementation does not include hardware forwarding or stall detection.

Expected Output

R0 =  0

R1 = 10

R2 = 20

R3 = 25

R4 = 30

R5 = 55

# Register File

- 32 general-purpose 32-bit registers (Reg[0:31])

- Reg[0] is treated as a zero register during decode (reads as 0)